SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE:CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Code of Conduct:

I R.Hashini/1924260352 (Name / Reg No) certify that this submission is my original work and that I have adhered to the guidelines specified for this assessment.

I understand that any violation of academic integrity rules will result in disciplinary action.

Signature of the student:

| Criteria | Weightage | Q1 10marks | Q2 10marks | Q3 10marks | Q4 10marks |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% |  |  |  |  |
| Method Selection | 20% |  |  |  |  |
| Solution Process | 25% |  |  |  |  |
| Accuracy of Calculation | 20% |  |  |  |  |
| Interpretation of Results | 15% |  |  |  |  |

Total Marks 		/40						Signature of the Faculty

Annexure A

ANALYTICAL PROBLEM SOLVING

1. A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Based on the given subnet masks, apply subnetting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.

Solution

Given: Block 192.168.20.0/24; Lab A = /26, Lab B = /27, Lab C = /28.

Method: Block size = 2^(32 − prefix); usable hosts = 2^(host bits) − 2 (network and broadcast addresses excluded). Subnets are allocated one after another (largest first) so that they do not overlap.

Step 1 – Block sizes: /26 → 2^6 = 64 addresses (mask 255.255.255.192); /27 → 2^5 = 32 addresses (mask 255.255.255.224); /28 → 2^4 = 16 addresses (mask 255.255.255.240).

Step 2 – Allocation: Lab A starts at .0 and ends at .63; Lab B starts at .64 and ends at .95; Lab C starts at .96 and ends at .111.

| Lab | Prefix / Mask | Network address | Valid host range | Broadcast address | Usable hosts |
| --- | --- | --- | --- | --- | --- |
| A | /26 255.255.255.192 | 192.168.20.0 | 192.168.20.1 – 192.168.20.62 | 192.168.20.63 | 2^6 − 2 = 62 |
| B | /27 255.255.255.224 | 192.168.20.64 | 192.168.20.65 – 192.168.20.94 | 192.168.20.95 | 2^5 − 2 = 30 |
| C | /28 255.255.255.240 | 192.168.20.96 | 192.168.20.97 – 192.168.20.110 | 192.168.20.111 | 2^4 − 2 = 14 |

Interpretation: The three subnets fit inside 192.168.20.0/24 without overlap and use 112 of the 256 addresses. The range 192.168.20.112 – 192.168.20.255 remains free for future labs. Lab A can host 62 devices, Lab B 30 and Lab C 14.

2.A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

Solution

Given: System 1 = 192.168.1.130, System 2 = 192.168.1.190, mask 255.255.255.192 (/26).

Method: Network address = IP AND subnet mask. Block size = 256 − 192 = 64, so the subnets in the last octet start at 0, 64, 128, 192. Broadcast = next network − 1.

Step 1 – System 1: 130 = 10000010, mask 192 = 11000000; AND = 10000000 = 128. Network = 192.168.1.128.

Step 2 – System 2: 190 = 10111110, mask 192 = 11000000; AND = 10000000 = 128. Network = 192.168.1.128.

| System | IP address | Network address | Valid host range | Broadcast address | IP in subnet? |
| --- | --- | --- | --- | --- | --- |
| 1 | 192.168.1.130 | 192.168.1.128 | 192.168.1.129 – 192.168.1.190 | 192.168.1.191 | Yes (128 ≤ 130 ≤ 191) |
| 2 | 192.168.1.190 | 192.168.1.128 | 192.168.1.129 – 192.168.1.190 | 192.168.1.191 | Yes (128 ≤ 190 ≤ 191) |

Subnet range and hosts: Subnet range for both systems = 192.168.1.128 – 192.168.1.191. Usable hosts in each subnet = 2^6 − 2 = 62.

Interpretation: Both systems belong to the same subnet, 192.168.1.128/26, so they can communicate directly without a router. Each subnet provides 62 usable host addresses. Note that 192.168.1.190 is the last usable host address of the subnet, just before the broadcast address 192.168.1.191.

3. An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

Solution

Given: Block 172.16.0.0/16; HR = /25, Sales = /26, IT = /27, Admin = /28.

Method: Block sizes are 2^(32 − prefix): /25 = 128, /26 = 64, /27 = 32, /28 = 16 addresses. The subnets are assigned consecutively from the start of the block, largest first, so there is no overlap and no wasted space between them. All the allocations fall in the last octet of 172.16.0.x.

| Department | Prefix / Mask | Network address | Valid host range | Broadcast address | Usable hosts |
| --- | --- | --- | --- | --- | --- |
| HR | /25 255.255.255.128 | 172.16.0.0 | 172.16.0.1 – 172.16.0.126 | 172.16.0.127 | 2^7 − 2 = 126 |
| Sales | /26 255.255.255.192 | 172.16.0.128 | 172.16.0.129 – 172.16.0.190 | 172.16.0.191 | 2^6 − 2 = 62 |
| IT | /27 255.255.255.224 | 172.16.0.192 | 172.16.0.193 – 172.16.0.222 | 172.16.0.223 | 2^5 − 2 = 30 |
| Admin | /28 255.255.255.240 | 172.16.0.224 | 172.16.0.225 – 172.16.0.238 | 172.16.0.239 | 2^4 − 2 = 14 |

Unused address range: The four subnets use 128 + 64 + 32 + 16 = 240 addresses (172.16.0.0 – 172.16.0.239). The remaining unused range is 172.16.0.240 – 172.16.255.255 (65,536 − 240 = 65,296 addresses).

Interpretation: Each department receives a subnet sized to its needs (126, 62, 30 and 14 hosts), and a very large portion of 172.16.0.0/16 remains available for future departments.

4. A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

Solution

Given: 172.16.10.0/24 subnetted with /27 (mask 255.255.255.224).

Step 1 – Number of subnets: Bits borrowed = 27 − 24 = 3, so subnets = 2^3 = 8.

Step 2 – Hosts per subnet: Host bits = 32 − 27 = 5, so usable hosts = 2^5 − 2 = 30. Block size = 256 − 224 = 32.

| Subnet no. | Network address | Valid host range | Broadcast address |
| --- | --- | --- | --- |
| 1 | 172.16.10.0 | 172.16.10.1 – 172.16.10.30 | 172.16.10.31 |
| 2 | 172.16.10.32 | 172.16.10.33 – 172.16.10.62 | 172.16.10.63 |
| 3 | 172.16.10.64 | 172.16.10.65 – 172.16.10.94 | 172.16.10.95 |
| 4 | 172.16.10.96 | 172.16.10.97 – 172.16.10.126 | 172.16.10.127 |
| 5 | 172.16.10.128 | 172.16.10.129 – 172.16.10.158 | 172.16.10.159 |
| 6 | 172.16.10.160 | 172.16.10.161 – 172.16.10.190 | 172.16.10.191 |
| 7 | 172.16.10.192 | 172.16.10.193 – 172.16.10.222 | 172.16.10.223 |
| 8 | 172.16.10.224 | 172.16.10.225 – 172.16.10.254 | 172.16.10.255 |

Step 3 – Subnet of 172.16.10.78: 78 lies between 64 and 95, so it belongs to subnet 3. Network address = 172.16.10.64; valid hosts = 172.16.10.65 – 172.16.10.94; broadcast address = 172.16.10.95. (Check: 78 AND 224 = 64.)

Step 4 – Is 172.16.10.95 in the same subnet? Yes, it lies within the range 172.16.10.64 – 172.16.10.95, so it is in the same subnet as 172.16.10.78. However, 172.16.10.95 is the broadcast address of that subnet, so it cannot be assigned to a host.

Interpretation: The /27 mask creates 8 subnets of 30 usable hosts each. 172.16.10.78 is a valid host address in 172.16.10.64/27, whereas 172.16.10.95 belongs to the subnet range but is reserved for broadcast.

5.A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

Solution

Given: 2001:0db8:0000:0000:0000:ff00:0042:8329 with prefix length /64.

Step 1 – Compression: (a) Remove leading zeros in every group: 2001:db8:0:0:0:ff00:42:8329. (b) Replace the longest run of consecutive all-zero groups (three groups) with "::" once: 2001:db8::ff00:42:8329.

Step 2 – Expansion: The compressed address has 5 groups, so "::" stands for 8 − 5 = 3 zero groups. Restoring them and padding each group to 4 hex digits gives 2001:0db8:0000:0000:0000:ff00:0042:8329, which matches the original.

Step 3 – Network prefix (/64): The first 64 bits are the first 4 groups: 2001:0db8:0000:0000. The network prefix is 2001:db8::/64 (host portion = ::ff00:42:8329 in the last 64 bits).

Step 4 – Number of host addresses: Host bits = 128 − 64 = 64, so the number of addresses = 2^64 = 18,446,744,073,709,551,616 (about 1.8 × 10^19). IPv6 has no broadcast address, so no addresses are subtracted for broadcast.

Interpretation: A /64 is the standard IPv6 subnet size. It gives each LAN an enormous host space, and the compressed form 2001:db8::ff00:42:8329 is much easier to read and write than the full notation.

RUBRICS FOR CALCULATION

| Criteria | Weightage | Level 4 (Exemplary) | Level 3 (Proficient) | Level 2 (Developing) | Level 1 (Beginning) |
| --- | --- | --- | --- | --- | --- |
| Problem Understanding | 20% | Clearly interprets problem, identifies all parameters and requirements accurately | Understands problem with minor gaps | Partial understanding; misses key elements | Misinterprets or unable to understand problem |
| Method Selection | 20% | Selects most appropriate method/formula with strong justification | Selects correct method with minor errors | Inappropriate or partially correct method | Unable to select method |
| Solution Process | 25% | Logical, systematic steps with correct approach throughout | Mostly correct steps with minor mistakes | Disorganized steps; multiple errors | No clear process |
| Accuracy of Calculation | 20% | All calculations accurate; correct final answer | Minor calculation errors | Frequent errors; incorrect final answer | No valid calculations |
| Interpretation of Results | 15% | Clearly interprets results with units and engineering relevance | Basic interpretation with minor omissions | Limited or unclear interpretation | No interpretation |